Four-sided LED escape sign luminaire for ceiling mounting according to DIN EN 60598-1, DIN EN 60598-2-22 and DIN EN 1838. 

Cube plastic luminaire with frameless bonnet and easy-to-mount bayonet catch. Due to the stable construction, the luminaire can also be suspended by chain or rope. 

Suitable for continuous or stand-by operation. Planning reliability thanks to logically printed pictograms according to DIN EN ISO 7010 and DIN ISO 3864 (left, right, bottom) as standard.

lbl-monitoring:

Version with automatic test system in accordance with DIN EN 62034 type S including SelfControl test button:

- Test results with fault analysis (illuminant, charge and battery circuit) and status displays (operation, function test, continuous function test) using 3 colour LEDs

- Automatic function test (weekly)

- Automatic continuous function test (every 12 months)

- Automatic charge monitoring

- Deep discharge protection with restart inhibit

- Idling and short-circuit shutdown of inverter

lbl-material: Plastic

lbl-color: RAL 9003

maße: 300 mm x 300 mm x 179.5 mm

~lbl-lv-diameter:  mm

lbl-mountingMethod: Decke

lbl-protectionClass: 2

lbl-protectionRating: IP 65

lbl-impactResistanceRate: IK 4

allowed-temperature-ds: 0 °C to 40 °C celsius

allowed-temperature-bs: 0 °C to 40 °C celsius

~lbl-lv-erkennungsweite: 30m m

lbl-pictogram: Set

lbl-articleNumber: WHXD413SC

lbl-zubehoer:

lbl-articleNumber: LFP3233-SET-2AKKU

~lbl-lv-manufacturer-addendum